TdRCB User Manual

TdRCB: An open-source software for estimating temperature-dependent receiver code bias variations

Innovation Academy for Precision Measurement Science and Technology,

Chinese Academy of Sciences©

Update: 2026.10

Contents

1 Overview	4

2 TdRCB software	4

2.1 Structure of TdRCB	4

2.2 Processing strategies	5

3 Requirements	7

3.1 Supported platforms	7

3.2 License	7

4 File structure	7

5 Configuration File (config.dat)	8

5.1 Input file configuration	8

5.2 Output file configuration	9

5.3 Processing strategies configuration	10

5.4 Station Information	11

6 Compile and Run	11

6.1 Compile	11

6.1.1 Windows	11

6.1.2	Linux	12

6.2 Run	13

6.2.1 day-by-day process	13

6.2.2 continuous processing	15

6.2.3 Linux Run	17

7 Output file format description	17

7.1 Result File	17

7.2 Process Information File	19

7.3 Log file	20

7.4 tfit File	21

7.5 tres File	21

7.6 Execution Interface	23

Appendix	24

Acknowledgement	27

References	27

1 Overview

Multi-system and Multi-frequency Receiver Code Bias Analysis Software (TdRCB) is an open-source GNSS data processing package specifically designed for extracting time-varying receiver code biases (RCBs). TdRCB is developed in C/C++, which can be easily ported to different operating systems, such as Windows and Linux. It is a post-processing software, which can process multi-frequency data from GPS, Galileo, BDS, GLONASS. Using a least-squares filter (LSF) based on undifferenced and uncombined observations to estimate time-varying RCBs. Furthermore, it requires observation files and broadcast ephemeris, offering a simple operation process. Processing strategies for different data can be configured in the formatted configuration file.

2 TdRCB software

2.1 Structure of TdRCB

Generally, the processing can be summarized into three parts.

（1）Data preparation. The required input files include the observation file and the broadcast ephemeris file. The processing parameters should then be specified in the configuration file. Temperature data are optional.

（2）Data processing. In this step, TDRCB first calculates the satellite information. Cycle slip detection is then performed using the geometry-free combination and the Melbourne-Wübbena combination. After finishing cycle slip detection, the LSF is employed to estimate unknown parameters. Finally, quality control is performed using the DIA method.

（3）Information output. In this step, processing information and estimated parameters, including satellite clock offsets, satellite elevation and azimuth angles, post-fit code and carrier-phase residuals, and time-varying receiver code biases (RCBs), are written to formatted output files. The log file primarily records satellite exclusions during data processing. If temperature data are provided, the software also outputs the fitted model describing the relationship between RCBs and temperature, along with the corresponding fitting residuals.

Fig. 1 The processing framework of TdRCB software

2.2 Processing strategies

For the processing strategies. The frequency-independent parameters, including the range, receiver clock offsets, satellite clock offsets, and tropospheric delays, were grouped into a single parameter and estimated as white noise. Ionospheric delays, ambiguities, and time-varying receiver code biases (RCBs) were also estimated. The detailed processing strategies are summarized in Table 1, and the complete mathematical derivations are provided in the Appendix for reference. Users can adjust the processing strategies according to their specific requirements.

Table 1 Processing strategies of TdRCB

| Item | Processing strategies |
| --- | --- |
| Satellite constellation | GPS/GLONASS/BDS/Galileo/QZSS |
| Frequency number | Dual-frequency and above (up to 5 frequencies) |
| Functional mode | Uncombined model |
| Sampling rate | 30s |
| Cut-off mask angle | 10° |
| Observation weight | P = sin2(E) /σ2(σ2 indicates variance of code or carrier phase) A priori precision of 0.003 and 0.3 m for raw phase and code |
| Filtering method | Least square filter |
| Frequency-independent parameters | Estimated as white noise |
| Ionospheric delays | Estimated as white noise |
| Phase ambiguities | Estimated as float constants for each satellite and frequency |
| SCB | Estimated as day constants for each satellite on the third or higher frequency band |
| RCB | Estimated as white noise |
| IFB | GLONASS, Estimated as white noise |

Table 2 lists the frequency priority order implemented in the code.

Table 2 Frequency priorities for each system

| GNSS Frequency | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| GPS | P1 | P2 | P5 | \ | \ |
| GLONASS | R1 | R2 | \ | \ | \ |
| Galileo | E1 | E5a | E5b | E6 | E5 |
| QZSS | P1 | P2 | P5 | \ | \ |
| BDS-2 | B1I | B2I | B3I | \ | \ |
| BDS-3 | B1I | B3I | B2b | B1C | B2a |

3 Requirements

3.1 Supported platforms

TdRCB software was developed in the C/C++ language using the cross-platform CMAKE build system. It can be compiled and executed on both Windows and Linux operating systems. It is recommended to compile and debug TdRCB under Visual Studio 2022 in Windows or using GCC/Clang in Linux. Computer configuration requirements are as follows:

Operating system: Linux or Windows

System type: 32 or 64 bit

Memory: at least 512MB

Hard disk space: at least 500MB

CMAKE Version: CMAKE version higher than 3.10

Visual Studio Version: at least version 2022

The software has been tested under Windows 10 and Ubuntu 16.04 or higher, with all tests passed.

3.2 License

-------------------------------------------------------------------------------------------------

Copyright (C) 2026 by APM, CAS, All rights reserved.

-------------------------------------------------------------------------------------------------

This program is an open-source software: you can redistribute it and/or modify it under the terms of the GNU General Public License as published by the Free Software Foundation.

4 File structure

The project directory structure is as follows.

Fig. 2 File list

Fig. 3 The structure of File

5 Configuration File (config.dat)

The configuration file mainly contains four parts:

- Input file configuration

- Output file configuration

- Processing strategies configuration

- Station names

5.1 Input file configuration

Users should specify the input file paths in conf/config.dat. All paths are interpreted relative to the current working directory from which TDRCB is executed. Depending on the processing mode, users should configure either individual file paths or directory paths. The applicable parameters are controlled by Continue:

- Continue = 0 (daily processing): Use OBS_PATH and NAV_PATH.

- Continue = 1 (continuous/multi-day processing): Use OBS_DIR and NAV_DIR.

（1）OBS_PATH

Specifies the path to a single-station RINEX observation file. This parameter is used only when Continue=0.

（2）NAV_PATH

Specifies the path to the broadcast ephemeris file. This parameter is used only when Continue = 0.

（3）TEMP_PATH

Specifies the path to the temperature data file. This parameter is optional. If left empty, temperature-related processing is skipped, and the .tfit and .tres output files are not generated. The temperature data file must meet the following requirements:

- File format: CSV text file, with columns separated by commas or tabs.

- Column order: Year, month, day, hour, minute, second, and temperature.

- Temperature unit: Degrees Celsius (°C).

- Sampling interval: 30 s, covering a complete 24-hour period (2,880 epochs).

- Missing values: Missing temperature values can be left blank and will be interpolated by the software.

（4）OBS_DIR

Specifies the directory containing multi-day observation files. This parameter is used only when Continue = 1. The software scans this directory for observation files.

（5）NAV_DIR

Specifies the directory containing broadcast ephemeris files for multiple days. This parameter is used only when Continue = 1. The software scans this directory for broadcast ephemeris files.

Fig. 4 Input file configuration

5.2 Output file configuration

Users should specify the output paths in conf/config.dat. All paths are interpreted relative to the current working directory from which TDRCB is executed.

（1）OUT_PRO

Specifies the directory for intermediate processing information. The output files use the .pro extension. The files contain information such as satellite positions, satellite clock offsets, ambiguities, elevation angles, and azimuth angles. For details, see Section 7.1.

（2）OUT_PATH

Specifies the directory for the main output files. The following three types of files are written to this directory:

| Extension | Description |
| --- | --- |
| .out | Epoch-wise receiver code bias (RCB) estimates. |
| .tfit | Quadratic fitting model describing the relationship between temperature and RCB. Generated only when TEMP_PATH is configured. |
| .tres | Fitting residuals, calculated as the estimated RCB minus the model-predicted RCB. Generated only when TEMP_PATH is configured. |

（3）OUT_LOG

Specifies the directory for log files, which use the .log extension. The log files record satellites excluded during processing and the corresponding reasons for exclusion, such as insufficient satellite elevation or an insufficient number of available frequency observations.

Fig. 5 Output file configuration

5.3 Processing strategies configuration

（1）UseBDS

| Value | Description |
| --- | --- |
| 0 | Use BDS-3 satellites only. |
| 1 | Use BDS-2 satellites only. |
| 2 | Use both BDS-2 and BDS-3 satellites. The first two frequencies are standardized as B1I and B3I for both systems. |

（2）Frequency

Specifies the number of frequencies used in processing. For example, 2 indicates dual-frequency processing.

（3）Exclsats

Specifies the PRNs of satellites to be manually excluded from processing. Multiple satellite PRNs should be separated by spaces.

（4）System

Specifies the GNSS constellations included in the solution.

（5）Continue

Specifies the processing mode:

- 0: Daily processing.

- 1: Continuous or multi-day processing.

Fig. 6 Processing options 1 configuration

5.4 Station Information

Fig. 7 Station information configuration

6 Compile and Run

6.1 Compile

6.1.1 Windows

Ensure that the complete compilation tool and CMAKE (version 3.10 or higher) is installed.

- Extract the files downloaded from GitHub

Fig. 8 The extracted files

- Search "x64 Native Tools Command Prompt for VS 2022"

Fig. 9 Open Command Prompt

- Navigate to Project Directory and Compile. Execute these commands in sequence

① cd your_project_path ②mkdir build ③cmake -B build ④cmake --build build

Fig. 10 Finish compile under Windows

    - Linux

Ensure that the complete compilation tool and CMAKE (version 3.10 or higher) is installed. Execute these commands in sequence: ① cd your_project_path ② mkdir build ③ cd build ④ cmake .. ⑤ make

Fig. 11 Finish compile under Linux

6.2 Run

Using Windows as an example, we will introduce the operational process with two examples: the day-by-day process and continuous processing

6.2.1 day-by-day process

（1）Compile code

Reference the section 6.1.

（2）Data Preparation

In the config.dat file, configure the parameters by setting the input and output paths according to your specific needs and file locations.

Fig. 12 Paths Requiring Configuration

（3）Run code

Recommend running in the command line: ① cd ./build/Debug ② TdRCB.exe

Fig. 13 Runtime Display

（4）Visualization

Two simple visualization scripts are located in the .\script directory and require MATLAB (version R2020b or higher) to run.

- RCB_day_by_day.m: Visualizes RCBs for a single day. Simply configure the file paths and specify the frequencies and systems to visualize.

Fig. 14 Visualizes RCBs for a single day

Fig. 15 Visualization Results

6.2.2 continuous processing

（1）Compile code

Reference the section 6.1.

（2）Data Preparation

In the config.dat file, configure the parameters by setting the input and output paths according to your specific needs and file locations. When use continuous processing, the following parameters do not need to be set and can be left as default: OBS_PATH, NAV_PATH, DOY. Configure the following input and output parameters:

Fig. 16 Paths Requiring Configuration

（3）Run code

Recommend running in the command line.

（4）Visualization

- RCB_Continues.m: Visualizes RCBs for continues day. Simply configure the file paths and specify the frequencies and systems to visualize.

Fig. 17 Visualizes RCBs for continues day

Fig. 18 Visualization Results

6.2.3 Linux Run

If you want run on the Linux system.

Execute these commands in sequence: ./TdRCB

Fig. 19 Run on Linux

7 Output file format description

7.1 Result File

Name: XXXX_GEC_2_YYYYDOY_RCB.out

Fig. 20 Structure of output file name

Table 3 Result output file format description

| Items |  | Value Type | Range |
| --- | --- | --- | --- |
| Epoch Time (GPST) | Year Month Day Hour Minute Second | char char char char char char | 1-4 6-7 9-10 12-13 15-16 18-22 |
| Receiver Position (SPP) | X (m) Y (m) Z (m) | double double double | 25-37 41-52 55-67 |
| Number of valid satellites |  | int | 71-72 |
| GDOP |  | double | 76-81 |
| GPS Time-varying RCB | P1 P2 P5 | double double double | 86-91 95-100 104-109 |
| GLONASS Time-varying RCB | R1 R2 | double double | 115-120 124-130 |
| Galileo Time-varying RCB | E1 E5a E5b E6 E5 | double double double double double | 136-141 146-151 157-162 167-172 177-182 |
| QZSS Time-varying RCB | P1 P2 P5 | double double double | 190-195 199-204 208-213 |
| BDS-3 Time-varying RCB | B1I B3I B2b B1C B2a | double double double double double | 218-223 230-235 242-248 255-260 266-271 |
| BDS-2 Time-varying RCB | B1I B3I B2I | double double double | 218-223 230-235 242-248 |

Fig. 21 Screenshot of RCBs result file

7.2 Process Information File

Name: XXXX_GEC_2_YYYYDOY_Inf.pro

Table 4 Process information file description

| Items |  | Value Type | Range |
| --- | --- | --- | --- |
| Satellite Sys. & PRN |  | int | 1-4 |
| Satellite Positions | X (m) Y (m) Z (m) | double double double | 6-13 23-35 38-51 |
| Satellite Clock Offset | (m) | double | 56-67 |
| Ion Delays | (m) | double | 75-83 |
| Ambiguity | F1 (cycle) F2 (cycle) F3 (cycle) F4 (cycle) F5 (cycle) | double double double double double | 91-99 107-115 124-131 140-147 156-163 |
| Satellite Elevation and Azimuth | Elevation (°) Azimuth (°) | double double | 173-179 188-195 |
| post-fit carrier phase residuals | F1 (m) F2 (m) F3 (m) F4 (m) F5 (m) | double double double double double | 199-211 215-227 231-243 247-259 263-275 |
| post-fit code residuals | F1 (m) F2 (m) F3 (m) F4 (m) F5 (m) | double double double double double | 280-292 296-308 312-324 328-340 344-356 |
| SCB | F3 (m) F4 (m) F5 (m) | double double double | 365-372 381-388 397-404 |

Fig. 22 Screenshot of Process Information file

7.3 Log file

Name: XXXX_GEC_2_YYYYDOY_Sat.log

The log file mainly contains the record of satellite eliminations.

Fig. 23 Screenshot of Log file

7.4 tfit File

This file provides the quadratic fitting models relating temperature (T) (°C) to receiver code bias (RCB) (m) for each GNSS constellation and frequency.

| Field | Description |
| --- | --- |
| System | GNSS constellation: GPS, GLONASS, Galileo, QZSS, or BDS. |
| Band | Frequency identifier, such as G_P1 or BDS_B1I. |
| N | Number of epochs included in the fitting process. |
| a | Quadratic coefficient（m/℃²） |
| b | Linear coefficient（m/℃） |
| c | Constant term（m） |
| R2 | Coefficient of determination R2, indicating the goodness of fit. |
| Equation | RCB(T)=aT2+bT+c |

Fig. 24 Screenshot of .tfit file

7.5 tres File

This file contains the fitting residuals at each epoch.

Residual definition: residual=RCBsolved-RCBmodel

Units: RCB, RCBfit, and residual are expressed in meters (m), while temperature is expressed in degrees Celsius (°C).

| Field | Description |
| --- | --- |
| Time(GPST) | Epoch time in GPS Time (GPST), represented by year, month, day, hour, minute, and second. |
| System | GNSS constellation name. |
| Band | Frequency identifier. |
| T(degC) | Temperature at the corresponding epoch (°C). |
| RCB(m) | Estimated receiver code bias at the corresponding epoch (m). |
| RCBfit(m) | RCB predicted by the fitted temperature model (m). |
| residual(m) | Fitting residual, calculated as the estimated RCB minus the model-predicted RCB (m). |

Fig. 25 Screenshot of . tres file

7.6 Execution Interface

Fig. 27 Runtime Display RCBs

Appendix

(1) CDMA RCB Function Model

We commence with the undifferenced and uncombined GNSS observation equations, which are written as (Teunissen and Montenbruck, 2017)

&E[Pr,fs(i)]=ρrs(i)+μf⋅Ir,1s(i)+br,f-bfs&E[Lr,fs(i)]=ρrs(i)-μf⋅Ir,1s(i)+λfαr,fs#1

where E(.) denotes the expectation operator. Pr,fs and Lr,fs represent observed-minus-calculated code and phase observations of a satellite s tracked by a receiver r on the frequency f at the epoch i, respectively. The frequency-independent parameters in this, including the range, receiver clock offsets, satellite clock offsets, and tropospheric delays, were combined into a single parameter ρrs(i). the first-order slant ionospheric delay Ir,1s(i) on the first frequency linked to other frequencies by the coefficient μf=λf2/λ12 with λf being the wavelength, the RCB br,f and its satellite counterpart bfs, and the float ambiguity αr,fs=Nr,fs+δr,f-δfs containing the receiver phase bias (RPB) δr,f, the satellite phase bias δfs, and the integer ambiguity Nr,fs. We consider the parameters with an epoch index i as time-varying quantities and assume the parameters without the epoch index to be time constant.

Departing from the time-constant assumption of the RCB, we consider it a time-varying parameter and rewrite Eq. (1) as

&E[Pr,fs(i)]=ρrs(i)+μf⋅Ir,1s(i)+br,f(1)+br,f(i)-bfs&E[Lr,fs(i)]=ρrs(i)-μf⋅Ir,1s(i)+λfαr,fs#2

where

br,fi=br,fi-br,f1#3

denotes the time-differenced receiver bias between epoch i and the first epoch, but parameters in Eq. (2) are not uniquely estimable due to the rank deficient problem. For this issue, we turn to the S-system theory (Baarda, 1973; Teunissen, 1985), which constraints a minimum set of parameters and formulates a full-rank model estimating the linear functionals of the original parameters. To make it clear, we first decompose the receiver and satellite code biases on the first two frequencies as

.r,f=.r,IF+μf.r,GF#4

where the geometry-free combination of a frequency-dependent quantity is defined as

.GF=1μ2-μ1.2-.1#5

Inserting Eq. (4) into the code observations equation in Eq. (2) yields

&E[Pr,fs(i)]=ρrs(i)+μfIr,1s(i)+br,IF(1)+μfbr,IF(1)-μfbr,GFs+br,f(i)+br,f>2s&E[Lr,fs(i)]=ρrs(i)-μfIr,1s(i)+λfαr,fs#6

where

br,f>2s=[br,f(1)-br,IF(1)-uf⋅br,GF(1)]-(b,fs-uf⋅b,GFs)

denotes the combination of receiver and satellite code biases on the third frequency and above.The rank deficiency elimination for Eq. (6) becomes straightforward, as we can lump the parameters that have common coefficients:

E[Pr,fs(i)]=ρrs(i)+μfIr,1s(i)+br,f(i)+br,f>2s#7

where

&ρrs(i)=ρrs(i)+br,IF(1)&Ir,1s(i)=Ir,1s(i)+br,GF(1)-b,GFs#8

denote the frequency-independent parameter and ionospheric delays, respectively. When dealing with the phase observations in Eq. (2), we first replace ρrs(i) and Ir,1s(i) with ρrs(i) and Ir,1s(i), resulting in additional code biases. By lumping these biases with ambiguities, we obtain the full-rank observation equations as

&E[Pr,fs(i)]=ρrs(i)+μfIr,1s(i)+br,f(i)+br,f>2s&E[Lr,fs(i)]=ρrs(i)-μfIr,1s(i)+λfαr,fs#9

where

αr,fs=αr,fs+b,IFs-br,IF1/λf-μf⋅b,GFs-br,GF1/λf#10

denote the estimable parameters contain the RCB at the first epoch.

(2) FDMA RCB Function Model

GLONASS raw code and carrier phase observation equations as (Teunissen, 2019)

&E[Pr,fs(i)]=ρrs(i)+μfs⋅Ir,1s(i)+br,f-bfs+ksΔbr,f&E[Lr,fs(i)]=ρrs(i)-μfs⋅Ir,1s(i)+λfs(αr,fs+ksΔδr,f)#11

different from CDMA observation equations, there exist receiver code and phase biases that are linear functions of the satellite frequency channel number, Δbr,f and Δδr,f, respectively. The meanings of the remaining parameters are the same as in Eq. (1).

In particular, GLONASS satellites have different wavelengths, expressed as λfs. Furthermore, the coefficients of the ionospheric parameters μfs are also satellite-dependent. To simplify the coefficients of the ionospheric parameters, they are defined based on the frequency of each GLONASS R1 and R2 satellite as follows:

Ffs=Ff0+ksΔFf#12

where Ff0 is the center frequency, ΔFf is the frequency offset, ΔF1=9/16, ΔF2=7/16, ksis the frequency channel number of satellite s, and they satisfy |ks|≤13, ΔF1/F10=1/2848, ΔF2/F20=1/2848. Therefore, the wavelength λfs can be expressed as:

λfs=cFf0+ksΔFf=λf01+ksΔFfFf0=2848λf02848+ks#13

where λf0=c/Ff0 is the wavelength corresponding to the center frequency. According to Equation (13), the coefficients of the ionospheric parameters can be written as:

μfs=λfsλ1s2=2848λf02848+ks∙2848+ks2848λ102=μf#14

Therefore, Equation (11) can be simplified as:

&E[Pr,fs(i)]=ρrs(i)+μf⋅Ir,1s(i)+br,f-bfs+ksΔbr,f&E[Lr,fs(i)]=ρrs(i)-μf⋅Ir,1s(i)+λfs(αr,fs+ksΔδr,f)#15

Departing from the time-constant assumption of the RCB, we consider it a time-varying parameter and rewrite Eq. (14) as:

&E[Pr,fs(i)]=ρrs(i)+μf⋅Ir,1s(i)+br,f(1)+br,fi-bfs+ksΔbr,f(1)+ksΔbr,f(i)&E[Lr,fs(i)]=ρrs(i)-μf⋅Ir,1s(i)+λfs(αr,fs+ksΔδr,f)#16

where

&br,fi=br,fi-br,f1&Δbr,f(i)=Δbr,f(i)-Δbr,f(1)#17

Note that, apart from the IFBs, the structure of the GLONASS equations is entirely consistent with that of CDMA systems. Therefore, following the rank deficiency elimination procedure used for CDMA, the following equations are obtained:

&E[Pr,fs(i)]=ρrs(i)+μfIr,1s(i)+br,fi+ksΔbr,f(i)&E[Lr,fs(i)]=ρrs(i)-μfIr,1s(i)+λfs(αr,fs+ksΔδr,f)#18

where

&ρrs(i)=ρrs(i)+br,IF(1)-b,IFs+ksΔbr,IF(1)&Ir,1s(i)=Ir,1s(i)+br,GF(1)-b,GFs+ksΔbr,GF(1)&αr,fs=αr,fs+ksΔδr,f+b,IFs-br,IF1-ksΔbr,IF1λfs-μf⋅b,GFs-br,GF1-ksΔbr,GF1λfs#

Acknowledgement

We sincerely thank Mr. Tomoji Takasu, the author of RTKLIB software. His great work and open-source spirit help many researchers. Our software is built on the software framework of RTKLIB, and we are grateful to the whole RTKLIB community.

References

- Teunissen P. J. G., Montenbruck O (eds) (2017) Springer handbook of global navigation satellite systems. Springer International Publishing, Cham

- Baarda W (1973) S-transformations and criterion matrices. Netherlands Geodetic Commission

- Teunissen P. J. G. (1985) Zero order design: generalized inverses, adjustment, the datum problem and S-transformations. In: Optimization and design of geodetic networks. Springer Berlin Heidelberg, Berlin, Heidelberg, pp 11–55

- Teunissen, P. J. G. (2019). A new glonass fdma model. GPS solutions, 23(4), 100.